AgriTrace

Offline-First IoT & Blockchain-Enabled Farm-to-Fork Traceability

Product Requirements Document (PRD)

SIH Problem Statement 26232 | Ministry of Food Processing Industries (MoFPI)

| Document Item | Specification |
| --- | --- |
| Product | AgriTrace — Low-Cost IoT Blockchain Node & Farm-to-Fork Traceability Platform |
| SIH PS | SIH26232 |
| Category | Hardware |
| Theme | Agriculture, FoodTech & Rural Development |
| Primary users | Farmers/aggregators, logistics operators, processors, quality teams, auditors, consumers |
| Document status | Implementation-ready product requirements |
| Reference UI | Provided AgriTrace Monitoring, Traceability, Alerts and Analytics screens |

# 1. Executive Summary

AgriTrace is a low-cost, rugged, offline-first shipment traceability platform designed around SIH26232. The system combines an IoT sensor node with local cryptographic storage, intermittent network synchronization, MQTT-based messaging, a backend traceability service, and a decentralized-ledger anchoring/verification layer. The web dashboard provides a truck-centric operational view of food shipments and makes environmental telemetry, offline synchronization, alerts and data integrity visible to operators.

The central product principle is: sensor data must remain available and verifiable even when the shipment temporarily loses cellular connectivity. Data is therefore captured locally, sequenced, cryptographically protected, queued for sync, and automatically transmitted after connectivity returns. The ledger stores verifiable proofs/anchors rather than requiring every raw sensor sample to be placed directly on-chain.

## 1.1 Product Value Proposition

Affordable traceability for SMEs that cannot justify enterprise cold-chain hardware.

Offline-first operation for agricultural and rural routes with intermittent connectivity.

Tamper-evident telemetry using device-side cryptographic signing/hashing and chained records.

Automatic synchronization after reconnection using lightweight messaging.

Environmental monitoring for temperature, humidity and ethylene/gas, with GPS and device health telemetry.

Auditable farm-to-fork history with QR-based verification for downstream users.

Truck-centric visualization that maps each sensor/batch to a physical shipment compartment.

# 2. Problem Statement and Requirements Baseline

The supplied SIH26232 statement identifies two core market constraints: enterprise-grade traceability is expensive for SMEs, and standard loggers can lose connectivity in remote agricultural areas, producing blind spots. The expected solution explicitly requires a rugged, affordable, tamper-proof node; environmental logging; local cryptographic storage; energy harvesting; and automatic synchronization to a decentralized ledger after reconnection.

| Requirement from PS | Product interpretation | Acceptance signal |
| --- | --- | --- |
| Low-cost IoT node | Commodity MCU, sensors, cellular/GNSS, local storage and power management. | Prototype BOM stays within the team's target cost. |
| Rugged operation | Sealed enclosure, protected connectors, vibration-resistant mounting and environmental protection. | Node continues logging under representative moisture/dust/vibration tests. |
| Environmental logging | Temperature, humidity and ethylene/gas as core telemetry. | Timestamped readings are captured at configurable intervals. |
| Offline continuity | Local queue persists records when cellular/network is unavailable. | No sample loss across a controlled network outage. |
| Cryptographic storage | Signed/chained records and secure key storage where available. | Modified record fails verification. |
| Energy harvesting | Solar as primary practical prototype path; thermal harvesting as an optional extension. | Energy subsystem extends runtime and reports energy/battery state. |
| Lightweight sync | MQTT over TLS with batching and acknowledgements. | Queued records sync automatically after reconnection. |
| Decentralized ledger | Store/anchor verifiable hashes or Merkle roots; keep high-volume telemetry off-chain. | Dashboard can prove whether a record/batch matches its anchored proof. |

# 3. Goals, Non-Goals and Success Metrics

## 3.1 Goals

Deliver an end-to-end demonstrable prototype from physical sensor node to dashboard.

Maintain data integrity during offline periods.

Provide real-time monitoring when connectivity is available.

Provide an explicit sync-state model: Live, Offline/Queued, Syncing, Synced, Verification Failed.

Make shipment events and environmental excursions easy to audit.

Keep the architecture modular enough to support different sensor and ledger implementations.

## 3.2 Non-Goals for MVP

Full national-scale logistics optimization.

Replacing regulated food-safety certification or laboratory testing.

Putting every high-frequency raw sensor reading directly on a blockchain.

Building a public marketplace or unrelated commerce module.

Advanced predictive shelf-life modeling as a mandatory MVP feature.

## 3.3 Suggested Success Metrics

| Metric | Target for prototype | Measurement |
| --- | --- | --- |
| Data continuity | 100% of test samples retained through planned outage | Compare generated vs. received sequence numbers |
| Sync recovery | < 60 seconds after network recovery for a small queued batch | Device/backend timestamps |
| Integrity | 100% of deliberately modified test records detected | Verification test suite |
| Dashboard freshness | < 5 seconds for connected telemetry | MQTT publish to UI timestamp |
| Duplicate prevention | No duplicate sequence IDs accepted | Backend idempotency checks |
| Alert latency | < 10 seconds under normal connectivity | Sensor event to UI alert |
| Traceability | Complete batch event timeline | Create → assign → transit → exception → delivery |

# 4. Users and Roles

| Role | Primary needs | Key permissions |
| --- | --- | --- |
| Farmer / Aggregator | Create shipment, attach device, see shipment health | Create/manage own shipments and devices |
| Logistics Operator | Monitor active trucks and connectivity | View live telemetry, acknowledge operational alerts |
| Quality / Food Safety | Review excursions and evidence | View history, verification, reports and export |
| Processor / Receiver | Verify shipment integrity before intake | Scan QR, view provenance and integrity status |
| Auditor | Trace immutable event history | Read-only access to events, proofs and verification |
| System Admin | Fleet, thresholds, users and devices | Full configuration and device management |
| Consumer | Simple provenance and trust information | Public QR consumer view; no operational controls |

# 5. Product Scope and Information Architecture

The supplied reference screens show an AgriTrace sidebar with operational modules. For the SIH26232 MVP, the navigation should be streamlined to the modules below.

| Priority | Page | Purpose |
| --- | --- | --- |
| P0 | Dashboard | Fleet/shipment overview, live health, sync and critical exceptions. |
| P0 | Shipments | Create, assign, track and review shipments. |
| P0 | Devices | Register, provision, assign and monitor IoT nodes. |
| P0 | Monitoring | Live environmental telemetry and truck-centric visualization. |
| P0 | Traceability | Farm-to-fork event timeline, hashes, proofs and QR verification. |
| P0 | Alerts | Environmental, device, tamper and sync exceptions. |
| P1 | Analytics | Shipment/device trends and operational KPIs. |
| P1 | Reports | Exportable audit/quality reports. |
| P1 | Profile / Settings | User profile, thresholds, notification and organization settings. |
| External/Public | Consumer View | QR-accessible shipment provenance and integrity summary. |

Remove Marketplace from the SIH MVP unless a separate business requirement is introduced. Consumer View should be reached through QR verification rather than being a primary operations sidebar destination.

# 6. UI/UX Product Requirements

## 6.1 Design Direction

Use the supplied AgriTrace visual language: dark navy left navigation, white/light-gray workspace, restrained teal/green accents, rounded cards and compact data density.

Keep the interface clean and operational; color should communicate state, not decorate every component.

Use consistent status colors: green = healthy/verified, amber = warning/pending, red = critical/failed, blue = informational/sync.

Every critical metric should expose its threshold or state where practical.

Use responsive layouts for laptop/tablet; the operational dashboard is desktop-first.

## 6.2 Global Shell

Persistent left sidebar with AgriTrace logo, primary navigation and collapse control.

Top bar with page title/subtitle, global search for shipments/devices/batches, notification icon and user profile.

Persistent status affordance for connectivity/sync when relevant.

Toast notifications for successful sync, QR creation, device assignment and critical errors.

## 6.3 Dashboard

KPI cards: Active Shipments, Devices Online, Devices Offline, Open Alerts, Pending Sync Records, Verified Shipments.

Live shipment map with truck markers and route status.

Compact critical-alert panel.

Offline queue/sync health card showing queued records and last successful sync.

Shipment status distribution and recent events.

## 6.4 Monitoring Page

Top warning banner when environmental values are outside configured safe ranges.

Metric cards for Temperature, Humidity, Gas/Ethylene and Battery.

Telemetry history table with timestamp, values, sequence number and source state.

Source badges must distinguish LIVE, OFFLINE QUEUED, SYNCED and VERIFIED where applicable.

Truck visualization is the primary shipment-centric monitoring surface.

Reference 1 — Monitoring screen supplied by the user.

## 6.5 Truck Shipment Visualization

Use the provided truck WebP as the visual base layer. Do not split the truck into separate images. Render compartment readings as HTML/React overlays positioned relative to the cargo body.

| Overlay | Required fields | Interaction |
| --- | --- | --- |
| Cargo block | Batch ID, temperature, humidity, ethylene/gas, integrity state | Click opens detailed shipment/device panel |
| Truck header | Shipment ID, origin, destination, current state | Click opens shipment |
| Connectivity | Wi-Fi/cellular state, last seen | Click opens device status |
| Battery | Battery %, charging/harvesting state | Click opens device health |
| GPS | Lat/long or readable location, timestamp | Click centers route/map |
| Alert badge | Active alert count/severity | Click filters alerts |

Recommended cargo overlay pattern:

<div className="relative">
  <img src="/assets/truck1.webp" className="w-full h-auto" alt="Shipment truck" />
  <CargoContainer batch="AG-2401" temperature={4.2} humidity={78} ethylene={0.8} integrity="verified" />
  <CargoContainer batch="AG-2402" temperature={5.1} humidity={81} ethylene={1.0} integrity="warning" />
</div>

## 6.6 Traceability Page

Vertical event timeline: device assigned, shipment created, departure, telemetry exception, offline period, sync completed, delivery and verification.

Integrity panel showing current record hash, previous hash, sequence, verification result and ledger anchor.

QR code panel for shipment verification.

Record filtering by event type and time range.

A verification failure must be visually prominent and explain what failed without exposing secret keys.

Reference 2 — Traceability screen supplied by the user.

## 6.7 Alerts Page

Cards grouped by severity and status.

Alert types: HIGH_TEMP, TEMPERATURE_EXCURSION, HIGH_HUMIDITY, GAS_ALERT, DEVICE_OFFLINE, TAMPER_DETECTED, INTEGRITY_FAILED, SYNC_FAILED, LOW_BATTERY.

Actions: View Shipment, Acknowledge/Dismiss, Open Details.

Each alert includes device, shipment, observed value, threshold, timestamp and event/sequence reference.

Resolved alerts remain auditable.

Reference 3 — Alerts screen supplied by the user.

## 6.8 Analytics Page

Shipment volume trend.

Shipment status distribution.

Alert overview.

Device online/offline/uptime view.

Environmental excursion trend.

Average sync delay and offline duration.

Integrity verification success/failure counts.

Reference 4 — Analytics screen supplied by the user.

# 7. Functional Requirements

| ID | Requirement | Description |
| --- | --- | --- |
| FR-01 | Authentication & roles | Users authenticate and receive role-scoped access. |
| FR-02 | Shipment creation | Create shipment with origin, destination, product/batch, expected conditions and device. |
| FR-03 | Device registration | Register a unique node ID and provision credentials. |
| FR-04 | Device assignment | Associate a device with one active shipment at a time. |
| FR-05 | Telemetry ingestion | Accept timestamped temperature, humidity, ethylene/gas, battery, GPS and connectivity data. |
| FR-06 | Offline capture | Device stores readings locally when network is unavailable. |
| FR-07 | Sequence integrity | Every telemetry record has monotonic device sequence number. |
| FR-08 | Cryptographic protection | Records are signed/hashed and linked to the prior record. |
| FR-09 | MQTT sync | Device publishes queued records securely after reconnecting. |
| FR-10 | Idempotent ingestion | Backend rejects or safely ignores duplicate sequence IDs. |
| FR-11 | Alert engine | Evaluate configurable thresholds and device conditions. |
| FR-12 | Traceability events | Create auditable shipment lifecycle events. |
| FR-13 | Ledger anchoring | Anchor batch/record proofs to a decentralized ledger. |
| FR-14 | Verification | Recompute and verify hashes against stored proof/ledger anchor. |
| FR-15 | QR verification | Generate a QR that opens a shipment verification view. |
| FR-16 | Analytics | Aggregate shipment, device, alert and sync metrics. |
| FR-17 | Reporting | Export traceability and excursion evidence. |
| FR-18 | Audit log | Record user actions and system security events. |

# 8. IoT Hardware & Firmware Requirements

## 8.1 Recommended Prototype Stack

| Subsystem | Recommended technology | Role |
| --- | --- | --- |
| MCU | ESP32-S3 / ESP32-class MCU | Low-power processing, connectivity and sensor orchestration |
| Temperature/Humidity | SHT31/SHTC3-class sensor | Environmental telemetry |
| Ethylene/Gas | Ethylene-capable sensor module appropriate to the prototype | Food maturity/spoilage-related gas signal |
| GNSS | u-blox-class GNSS module | Shipment location |
| Cellular | LTE Cat-1/Cat-1 bis or NB-IoT/LTE-M module depending on coverage | Wide-area connectivity |
| Local storage | Industrial microSD or flash | Offline telemetry queue |
| RTC | Low-power RTC | Stable timestamping during connectivity loss |
| Tamper | Lid/reed switch + optional accelerometer | Physical tamper and shock/vibration detection |
| Power | Li-ion/LiFePO4 battery + BMS | Primary energy storage |
| Energy harvesting | Solar panel + charge controller | Runtime extension |
| Security | Secure element where practical | Device identity/key protection |
| Enclosure | Rugged sealed enclosure | Dust/moisture/vibration protection |

Exact sensor selection must be validated against the food/product type, required gas range, calibration needs and environmental conditions. The SIH statement establishes ethylene/gas monitoring as a requirement but does not prescribe a specific sensor model.

## 8.2 Firmware Responsibilities

Read sensors on a configurable interval.

Timestamp and sequence each sample.

Normalize/validate sensor values and attach quality flags.

Build a canonical telemetry record.

Hash/sign the record and link it to the previous record.

Persist the record before attempting network transmission.

Publish live telemetry when connected.

Queue unsent records during outage.

Retry with exponential backoff and batch uploads.

Report battery, charging/harvesting, signal and firmware state.

Detect tamper/open events and generate priority records.

Support secure configuration and firmware update strategy for production.

# 9. Offline-First & Synchronization Architecture

Offline-first is a primary product requirement, not an edge case.

Sensor → Validate → Sequence → Hash/Sign → Local Durable Queue
                                   ↓
                           Network Available?
                              /           \
                            No             Yes
                            ↓               ↓
                       Keep queued      MQTT/TLS batch
                                            ↓
                                      Backend ACK
                                            ↓
                                    Mark records synced
                                            ↓
                                  Ledger proof/anchor
                                            ↓
                                      UI verification

| State | Meaning | UI treatment |
| --- | --- | --- |
| LIVE | Record arrived directly from connected node. | Green LIVE badge |
| QUEUED | Record safely stored locally but not yet uploaded. | Amber offline badge |
| SYNCING | Queued records are being transmitted. | Blue sync indicator |
| SYNCED | Backend accepted the record. | Green/neutral synced state |
| VERIFIED | Cryptographic proof matches expected chain/anchor. | Green verification badge |
| FAILED | Integrity or synchronization failed. | Red alert + actionable details |

## 9.1 MQTT Topic Design

agritrace/{deviceId}/telemetry
agritrace/{deviceId}/events
agritrace/{deviceId}/status
agritrace/{deviceId}/ack
agritrace/{deviceId}/config

Use MQTT over TLS.

Use QoS appropriate to the criticality of telemetry; the prototype can use QoS 1 for delivery-at-least-once semantics.

Use message IDs/device sequence numbers for deduplication.

Never rely on MQTT delivery alone as the integrity mechanism; the record itself must be cryptographically protected.

# 10. Cryptographic Integrity Model

The integrity model should make a modified historical record detectable without storing sensitive private keys in the dashboard. A simple prototype can use a hash chain; production hardware should use a secure element or equivalent key-protection mechanism.

record_n = {
  device_id,
  shipment_id,
  sequence,
  timestamp,
  temperature,
  humidity,
  ethylene,
  latitude,
  longitude,
  battery,
  previous_hash
}

record_hash = SHA-256(canonical(record_n))
signature   = Sign(private_device_key, record_hash)

Canonical serialization must be deterministic so the same record produces the same digest.

The previous_hash field links records into a tamper-evident sequence.

The backend verifies sequence continuity and signature/hash before accepting a record.

Periodic Merkle roots or batch hashes can be anchored on-chain to reduce transaction cost.

The ledger should contain proof/anchor data, not secrets and not necessarily all raw telemetry.

# 11. Blockchain / Decentralized Ledger Layer

The dashboard requires a decentralized verification capability. For the MVP, the recommended pattern is off-chain telemetry with on-chain proof anchoring. This keeps high-frequency sensor traffic efficient while retaining verifiability.

| Layer | Data | Reason |
| --- | --- | --- |
| Device | Raw samples, local queue, hash/signature | Works offline and protects the source record |
| Backend DB | Telemetry, events, shipment metadata, hashes, sync state | Fast queries and dashboard analytics |
| Proof service | Batch hash/Merkle root and verification metadata | Build efficient integrity proofs |
| Ledger | Shipment/batch proof anchor, timestamp, reference | Independent tamper-evidence |
| Dashboard | Human-readable state and verification result | Operations and audit usability |

The exact ledger/network choice is an implementation decision and should be finalized based on SIH judging requirements, deployment cost, transaction model, and available infrastructure. The PRD does not assume that a particular chain is mandatory.

# 12. Backend Architecture

IoT Node
  │ MQTT/TLS
  ▼
MQTT Broker
  │
  ▼
Ingestion Service ──► Validation ──► Integrity Verification
  │                                      │
  │                                      ├──► Alert Engine
  │                                      ├──► Sync State
  │                                      └──► Proof/Anchor Service
  ▼
PostgreSQL / Time-Series Storage
  │
  ├── REST API / WebSocket
  ▼
React Dashboard

## 12.1 Recommended Software Stack

| Layer | Recommended stack | Responsibility |
| --- | --- | --- |
| Frontend | React + TypeScript + Vite | Dashboard UI |
| Styling | Tailwind CSS | Design system/layout |
| Charts | Recharts or Apache ECharts | Telemetry/analytics |
| Maps | Mapbox GL JS or Google Maps | GPS/routes |
| Backend | FastAPI + Python | REST/WebSocket and business logic |
| MQTT | EMQX/Mosquitto or managed broker | Device messaging |
| Database | PostgreSQL + PostGIS; optional TimescaleDB | Transactional + telemetry + geospatial data |
| Cache/queue | Redis | Short-lived state, jobs, rate limiting |
| Blockchain | EVM-compatible or other SIH-approved decentralized ledger | Proof anchoring/verification |
| Storage | Object storage | Reports/exports/firmware/artifacts |
| Deployment | Docker + cloud VM/container platform | Reproducible deployment |

# 13. Database Model

| Entity | Key fields |
| --- | --- |
| users | id, organization_id, name, email, role, status |
| devices | id, serial_number, firmware_version, public_key, battery, signal, status, last_seen |
| shipments | id, shipment_code, product, batch_code, origin, destination, status, created_at |
| device_assignments | device_id, shipment_id, assigned_at, unassigned_at |
| telemetry | id, device_id, shipment_id, sequence, timestamp, temp, humidity, ethylene, lat, lon, battery, hash, signature, sync_state |
| shipment_events | id, shipment_id, event_type, timestamp, location, metadata, hash |
| alerts | id, shipment_id, device_id, type, severity, observed_value, threshold, status, created_at, resolved_at |
| ledger_anchors | id, shipment_id, root_hash, transaction_id, network, anchored_at, verification_status |
| sync_batches | id, device_id, first_sequence, last_sequence, count, status, started_at, completed_at |
| audit_logs | id, actor_id, action, target_type, target_id, timestamp, metadata |

# 14. API Specification

| Method | Endpoint | Purpose |
| --- | --- | --- |
| POST | /api/v1/shipments | Create shipment |
| GET | /api/v1/shipments | List/filter shipments |
| GET | /api/v1/shipments/{id} | Shipment detail |
| POST | /api/v1/shipments/{id}/assign-device | Assign device |
| GET | /api/v1/devices | List devices |
| POST | /api/v1/devices | Register/provision device |
| GET | /api/v1/devices/{id}/telemetry | Telemetry history |
| GET | /api/v1/shipments/{id}/timeline | Traceability timeline |
| GET | /api/v1/shipments/{id}/verification | Integrity verification result |
| GET | /api/v1/alerts | Alert list |
| PATCH | /api/v1/alerts/{id} | Acknowledge/resolve alert |
| GET | /api/v1/analytics/overview | Dashboard KPI data |
| GET | /api/v1/shipments/{id}/qr | Generate/retrieve QR payload |
| POST | /api/v1/sync/telemetry | Batch sync endpoint for gateway/direct integration |

Use WebSocket/SSE for live dashboard updates where appropriate; REST remains the system-of-record API for queries and administrative operations.

# 15. Alert & Rule Engine

| Rule | Example | Severity |
| --- | --- | --- |
| Temperature high | temperature > configured upper bound | Critical/Warning |
| Temperature low | temperature < configured lower bound | Warning |
| Humidity high/low | humidity outside product range | Warning |
| Gas/ethylene | ethylene/gas signal above threshold | Critical |
| Device offline | No heartbeat for configured interval | Warning |
| Tamper | Enclosure opened or unexpected movement | Critical |
| Low battery | Battery below configured percentage | Warning |
| Integrity failed | Signature/hash verification mismatch | Critical |
| Sequence gap | Unexpected missing sequence range | Critical/Warning |
| Sync failed | Repeated MQTT/backend acknowledgement failure | Warning |

Thresholds must be configurable per product/batch because safe environmental ranges are product-dependent. The reference screen's example values should be treated as demo configuration, not universal food-safety limits.

# 16. QR / Consumer Verification

Each shipment receives a unique public verification identifier.

QR resolves to a public consumer/auditor page.

Show product/batch, origin, shipment dates, major checkpoints, environmental compliance summary and integrity status.

Do not expose device private keys, internal credentials, personal information or raw infrastructure secrets.

Verification page should explain whether the displayed history is verified, partially verified, pending synchronization or failed.

# 17. Security Requirements

TLS for MQTT and HTTPS for APIs.

Per-device identity and credential provisioning.

Private keys never stored in frontend code.

JWT/OAuth2-style session management with role-based authorization.

Server-side validation for all telemetry and administrative inputs.

Idempotency and sequence checks to prevent duplicate/replayed samples.

Rate limiting and broker ACLs per device/topic.

Secrets stored in a secret manager/environment configuration, never committed to source control.

Audit logs for user actions and integrity events.

Database backups and retention policy.

Signed firmware/update mechanism should be planned for production.

# 18. Performance & Reliability Requirements

| Area | Requirement |
| --- | --- |
| Live telemetry | UI should reflect connected sensor updates within the agreed latency target. |
| Offline queue | No data loss during planned connectivity outages within device storage capacity. |
| Scalability | Backend must process batched telemetry without blocking dashboard queries. |
| Availability | Critical ingestion path should degrade gracefully if analytics/ledger services are temporarily unavailable. |
| Consistency | Shipment/device assignment must be transactional. |
| Observability | Logs, metrics and health checks for broker, API, DB, sync and ledger services. |
| Recovery | Queued data must resume automatically after reconnection. |

# 19. Testing & Acceptance Plan

| Test category | Key tests |
| --- | --- |
| Sensor | Accuracy sanity checks, invalid values, disconnected sensor behavior |
| Offline | Disable cellular/network, generate records, restore network, verify complete ordered sync |
| Integrity | Modify a stored record and confirm verification failure |
| Replay | Resend same sequence and confirm idempotent handling |
| Gap | Delete/drop one sequence and confirm gap detection |
| MQTT | Broker disconnect/reconnect, retained state, QoS behavior |
| Alerts | Cross thresholds, resolve condition, confirm alert lifecycle |
| GPS | Valid fix, unavailable fix, stale location |
| Battery | Low-battery warning and charging/harvesting state |
| Tamper | Open enclosure / trigger sensor and confirm alert |
| Blockchain | Anchor proof, verify proof, simulate mismatch |
| UI | Responsive layout, loading, empty, offline, error and permission states |
| Security | Unauthorized API calls, expired token, invalid signatures, malformed payloads |

# 20. MVP Delivery Plan

| Phase | Deliverables |
| --- | --- |
| Phase 1 — Foundation | Repo setup, design system, auth, PostgreSQL schema, device/shipment CRUD |
| Phase 2 — IoT | Sensor firmware, local queue, sequence/hash chain, device status |
| Phase 3 — Messaging | MQTT broker, secure ingestion, acknowledgements, sync state |
| Phase 4 — Monitoring | Monitoring page, telemetry history, truck overlay, GPS |
| Phase 5 — Alerts | Rule engine, alert UI, notifications and resolution flow |
| Phase 6 — Traceability | Event timeline, integrity verification, QR view |
| Phase 7 — Ledger | Proof batching, ledger anchoring, verification service |
| Phase 8 — Analytics | KPIs, charts, reports and operational metrics |
| Phase 9 — Hardening | Security, offline tests, hardware enclosure/power tests, demo rehearsal |

# 21. Recommended Repository Structure

agritrace/
├── apps/
│   ├── web/                    # React + TypeScript dashboard
│   └── api/                    # FastAPI backend
├── firmware/
│   └── node/                   # MCU firmware
├── services/
│   ├── ingestion/              # MQTT consumer / validation
│   ├── alerts/                 # Rules and notifications
│   ├── proof/                  # Hash/Merkle + ledger anchoring
│   └── sync/                   # Batch synchronization jobs
├── packages/
│   ├── types/                  # Shared schemas/types
│   └── crypto/                 # Canonicalization/integrity helpers
├── infra/
│   ├── docker/
│   └── mqtt/
├── docs/
│   ├── architecture/
│   ├── api/
│   └── hardware/
└── README.md

# 22. Key UI Data Contracts

TelemetryRecord {
  deviceId: string
  shipmentId: string
  sequence: number
  timestamp: string
  temperatureC: number | null
  humidityPct: number | null
  ethylenePpm: number | null
  latitude: number | null
  longitude: number | null
  batteryPct: number | null
  network: "online" | "offline" | "unknown"
  syncState: "live" | "queued" | "syncing" | "synced" | "failed"
  integrity: "verified" | "pending" | "failed"
}

# 23. Demo Scenario for SIH Presentation

Create a shipment for a food batch and assign an AgriTrace node.

Start the truck view with normal temperature, humidity and gas readings.

Show live GPS and battery/network state.

Intentionally disconnect cellular connectivity.

Continue generating sensor readings; the dashboard/device state changes to Offline/Queued.

Demonstrate that sequence numbers continue and data remains stored locally.

Reconnect the network.

Show automatic MQTT batch synchronization and transition from Queued → Syncing → Synced.

Generate a controlled temperature or gas excursion and show the Alerts page.

Show the traceability timeline recording the excursion and offline event.

Generate/verify the shipment QR.

Show cryptographic verification and ledger proof/anchor.

Open Analytics to show shipment, alert, device and synchronization metrics.

# 24. Risks and Mitigations

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Poor cellular coverage | Delayed sync | Durable local queue + batch retry + store-and-forward |
| Sensor drift | False alerts | Calibration procedure + sensor health flags + product-specific thresholds |
| Battery depletion | Node stops logging | Solar harvesting + low-power firmware + battery alerts |
| Ledger outage | Proof delayed | Keep verified backend records and queue proof anchoring asynchronously |
| Duplicate messages | Duplicate telemetry | Device sequence IDs + backend idempotency |
| Tampering | Loss of trust | Secure keys + hash chain + tamper event + ledger anchor |
| Overloading blockchain | High cost/latency | Anchor batches/Merkle roots instead of raw telemetry |
| UI complexity | Poor operator usability | Truck-first visualization + concise cards + drill-down details |

# 25. Open Decisions Before Hardware Freeze

Final ethylene/gas sensor and calibration method.

Final cellular technology based on target Indian deployment areas.

Exact energy-harvesting architecture and expected duty cycle.

Enclosure ingress-protection target and mechanical test plan.

Final decentralized ledger/network selected for the SIH demonstration.

Whether GNSS is always-on or duty-cycled based on movement.

Data retention period and sampling frequency per product class.

Exact environmental thresholds for each food category.

Production-grade secure element and device provisioning workflow.

# 26. Final Product Definition

AgriTrace is not merely a dashboard. The product is an end-to-end trust pipeline: the physical node captures evidence, the offline-first firmware preserves that evidence during connectivity loss, the MQTT/backend layer transports and validates it, the integrity layer makes modification detectable, the ledger provides independent proof anchoring, and the dashboard makes the result operationally understandable.

For the SIH26232 prototype, implementation should prioritize a convincing end-to-end demonstration over unnecessary feature breadth. The highest-value flow is a real sensor reading → offline storage → network interruption → automatic sync → alert/traceability event → cryptographic verification → ledger proof → truck-centric dashboard/QR verification.

# Appendix A — Reference Asset Usage

| Asset | Use in product |
| --- | --- |
| Provided Monitoring screenshot | Reference for environmental metric cards and telemetry history |
| Provided Traceability screenshot | Reference for event timeline, integrity panel and QR verification |
| Provided Alerts screenshot | Reference for severity-based alert cards and actions |
| Provided Analytics screenshot | Reference for KPI cards, charts and device/shipment status |
| Provided truck WebP | Primary visual asset for truck-centric shipment monitoring |